# KASUN佳之讯液晶                 	            HTTP://WWW.SZOYN.COM

#  ZCM--1602A	      

*16Characters 2 lines* Controller LSI built-in * +5V single power supply



LED:带LED 背光高度为MAX 13.0

# MECHANICAL DATA

| #### Module Size | 80.0×36.0×9.3 (LED:13.0) | # Display Area | 64.5×13.8 |
| --- | --- | --- | --- |
| ##### Character Pitch | 3.9W×5.05H | Character Size | 3.2W×4.35H |
| Display Format | 16 Characters×2 Lines | Driving Method | 1/16 Duty; 1/5 Bias |

| ## ABSOLUTE MAXIMUM RATINGS | # ELECTRICAL CHARACTERISTICS (Ta=25℃,VDD=5V±0.25V) |
| --- | --- |
| Power supply for Logic: 0~7V | Input ‘High’ Voltage: 2.2~VDD |
| Power supply for LCD Driver: 0~5V | Input ‘Low’ Voltage: -0.3~0.6 |
| Input Voltage: VSS~VDD | Output ‘High’ Voltage: 2.2(Min.) |
| Operating Temperature: 0~50℃ | Output ‘Low’ Voltage: 0.4(Max.) |
| Storage Temperature: -10~60℃ | Power supply Current: 3.0mA(Max.) |

PIN CONNECTION

| Pin | Symbol | Level | ### Function | Pin | Symbol | Level | ###### Function |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | VSS | 0V | Ground | 06 | E | - | H, H→L: Enable Signal |
| 02 | VDD | 5V | Power supply for Logic | 07~14 | DB0~DB7 | H/L | Data bus |
| 03 | V0 | - | Power supply for LCD Driver | 15 | VBLA | LED+ | For connection Backlight + |
| 04 | RS | H/L | H: Data; L: Instruction | 16 | VBLK | LED- | For connection Backlight - |
| 05 | R/W | H/L | H: Read; L: Write | - | - | - | - |

19