# KASUN佳之讯液晶                  HTTP://WWW.SZOYN.COM

#  ZCM--12232A      	      

*122x16dots 2 lines* Controller LSI built-in * +5V single power supply



| # MECHANICAL DATA |  |
| --- | --- |
| # Module Size: 65.8×27.2×8.5 (5.0) | Effective Display Area: 54.8×18.3 |
| Dots Pitch: 0.4×0.45 | Dots Size: 0.36×0.41 |
| # Display : 122×16 Dots×2Lines | # Driving Method: 1/32 Duty |

| # ABSOLUTE MAXIMUM RATINGS | # ELECTRICAL CHARACTERISTICS (Ta=25℃,VDD=5V±0.25V) |
| --- | --- |
| Power supply for Logic: 0~8V | Input ‘High’ Voltage: (VSS+2.0)~ADD |
| Power supply for LCD Driver: 0~16.5V | Input ‘Low’ Voltage: VSS ~(VSS+0.8) |
| Input Voltage: (VSS-0.3)~(VDD+0.3) | Output ‘High’ Voltage: (VSS+2.4)<Min> |
| Operating Temperature: 0~50℃ | Output ‘Low’ Voltage: (VSS+0.4)<Max> |
| Storage Temperature: -20~70℃ | Power supply Current: 1.2mA: |

PIN CONNECTION

| Pin | Symbol | Lever | Function | Pin | Symbol | Lever | Function |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | VDD | 5V | Power supply for Logic | 07 | R/W | H/L | H: Read; L: Write |
| 02 | VSS | 0V | Ground | 08 | A0 | H/L | H: Data; L: Instruction |
| 03 | V0 | - | Power supply for LCD | 09~16 | DB0~DB7 | H/L | Data bus |
| 04 | RST | H/L | System Reset | 17 | NC | - | No Connection |
| 05 | E1 | H/L | Enable Signal (Slave) | 18 | NC | - | No Connection |
| 06 | E2 | H/L | Enable Signal (Master) | - | - | - | - |

32