# KASUN佳之讯液晶                  HTTP://WWW.SZOYN.COM

#  ZCM--12864A

*128x64Dots * Controller LSI built-in *



LED:带LED 背光高度为MAX 13.0

| # MECHANICAL DATA |  |
| --- | --- |
| # Module Size: 113.0×65.0×8.9 (LED:13.0) | Effective Display Area: 73.4×38.8 |
| Dots Pitch: 0.508×0.508 | Dots Size: 0.458×0.458 |
| # Display : 128×64 Dots | # Driving Method: 1/64 Duty |

| # ABSOLUTE MAXIMUM RATINGS | # ELECTRICAL CHARACTERISTICS (Ta=25℃,VDD=5V±0.25V) |
| --- | --- |
| Power supply for Logic: 0~7V | Input ‘High’ Voltage: 0.7VDD <Min> |
| Power supply for LCD Driver: 0~19.0V | Input ‘Low’ Voltage: 0.3VDD <Max> |
| Input Voltage: VSS~VDD | Output ‘High’ Voltage: 2.4V <Min> |
| Operating Temperature: 0~60℃ | Output ‘Low’ Voltage: 0.4V <Max> |
| Storage Temperature: -20~70℃ | Power supply Current: 4.0mA: |

PIN CONNECTION

| Pin | Symbol | Lever | Function | Pin | Symbol | Lever | Function |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | VSS | 0V | Ground | 15 | CS1 | H | Chip select Signal for IC1 |
| 02 | VDD | 5V | Power supply for Logic & LCD (+) | 16 | CS2 | H | Chip select Signal for IC2 |
| 03 | V0 | - | Power supply for LCD (Variable) | 17 | RST | H/L | System Reset |
| 04 | D/I | H/L | H: Data; L: Instruction | 18 | VEE | H/L | Negative Voltage for LCD Driving |
| 05 | R/W | H/L | H: Read; L: Write | 19 | VBLA | LED+ | For connection Backlight + |
| 06 | E | - | H, H→L: Chip Enable Signal | 20 | VBLK | LED- | For connection Backlight - |
| 07~14 | DB0~DB7 | H/L | Data bus | - | - | - | - |

47